- abrir um novo arquivo 3D3 e configurar como os anteriores;

- fazer uma circunferência com centro de Coordenadas = 30,30 e Raio = 30;

- traçar com Polyline duas retas paralelas e distantes do centro de 20 unidades, em y, do centro;

- editar até obter a figura da tela e usando _pedit  transformar as duas meias luas em entidades únicas;

- Command: _extrude

-      Select objects: 1 found (clique uma das meia-luas)

-      Select objects: 1 found (clique a outra)

-      Select objects: <enter>

-      Path/<Height of Extrusion>: 40 <enter>

- Extrusion taper angle <0>: <enter>

Command: _ucs (colocar o UCS no centro das meias-luas)

- Origin/ZAxis/3point/OBject/View/X/Y/Z/Prev/Restore/Save/Del/?/<World>: o

- Origin point <0,0,0>: _cen of (clique com o Osnap Center ativo uma das meia-luas)

- Command: _cylinder

- Elliptical/<center point> <0,0,0>:  <enter> (o centro do cilindro é o UCS)

- Diameter/<Radius>: 30 <enter>

- Center of other end/<Height>: 40 <enter>

Command: _union

Select objects: 1 found (clique uma das laterais em meia-lua)

Select objects: 1 found (clique a outra)

Select objects: 1 found (clique o cilindro)

Select objects: <enter>

Command: _rotate3d

Select objects: 1 found (clique o sólido final)

Select objects: <enter>

Axis by Object/Last/View/Xaxis/Yaxis/Zaxis/<2points>: x <enter>

Point on X axis <0,0,0>: <enter>

<Rotation angle>/Reference: 180 <enter> (o sólido gira em torno do eixo x de 180)